LB-6090、1212/1218/1224 三维广告雕刻机

性能特点:

◆机身采用厚壁钢管焊接，经巨型龙门刨处理，确保机床精度高，钢性强

◆整机采用进口高精度滚珠丝杆、导轨，运动平稳，精度更高

◆三轴均采用全防尘结构，确保机器使用寿命长久

◆机电全优设计，精选各种电气配件，将故障率降到最低

◆具有断点记忆、断电续雕加工时间预测等功能，保证在意外情况下加工

适用于：胸牌制作、建筑模型、亚克力切割、电子治具、标识业、电子教学等行业

Function Characterisitics:

1.Welded steel tube fuselage with thick, by giant planer processing to ensure high precision machine tools, steel strong;

2. machine adopts imported high-precision ball screw, rail, smooth motion, higher precision;

3.whole dust structure axes are used to ensure a long life of the machine;

4. excellent mechanical and electrical design, featuring a variety of electrical accessories, to minimize the failure rate;

5. with the breakpoint memory, power outages continued carving of processing time and other functions, to ensure processing in unexpected situations

Applies to: badge making, architectural models, acrylic cutting, electronic governance, identify industry, e-learning industry

Technincal parameter:

| 型号（Type） | LB-6090 | LB-1212/LB-1218 | LB-1224 |
| --- | --- | --- | --- |
| X/Y轴行程 xy axle distance | 600mm*880mm | 1200mm*1200mm/ 1200mm*1800mm | 1200mm*2400mm |
| Z 轴行程 z axle distance | 150mm | 150mm | 150mm |
| 最大速度 Maximum speed | 6.0(m/min) | 10.0(m/min) | 10.0(m/min) |
| 分辨率 Differentiate effciency | 0.005mm | 0.05mm | 0.05mm |
| 处理器 Processing Unit | PCI/DSP | PCI\DSP | PCI\DSP |
| 传动方式 Transmission belt | 丝杆传动 Screw driver | 丝杆传动 Screw driver | 丝杆传动 Screw driver |
| 驱动方式 Driving style | 步进 Step | 步进 Step | 步进 Step |
| 导轨 Guide | 圆轨 Circular guide | 圆轨 Circular guide | 圆轨 Circular guide |
| 接口界面 Interface screen | Win/USB | Win/USB | Win/USB |
| 软件运行环境 Software working condition | Windows2000/XP/98 | Windows2000/XP/98 | Windows2000/XP/98 |
| 雕刻指令 Carving orders | G-code/HP-GL | G-code/HP-GL | G-code/HP-GL |
| 功耗(不含主轴) Power consume(No major axle | 1000W | 1000W | 1200W |
| 主轴 Major axle | 1500w(water-colded) | 1500w(water-colded) | 2200w(water-colded) |
| 主轴转速 Major axle speed | 0-24000(circle/min) | 0-24000(circle/min) | 0-24000(circle/min) |
| 刀具径 Carving diameter | 3.175mm/4mm/6mm | 3.175mm/4mm/6mm | 3.175mm/4mm/6mm/8mm/10mm/12.7mm |
| 电源 Power source | 220V50HZ | 220V50HZ | 220V50HZ |